H.Snel, G.P.Corten, J.W.M.Dekker, M.Francke, J.d.Boer, P.Chaviaropoulos, S.Voutsinas, "Progress in the Joule Project: Multiple Stall Levels", Proc. EWEC ’99, pp 142-145, 1-5 March 1999, Nice, France.

Abstract

Multiple stall is defined as the occurrence of more than one power level in stall at apparently identical conditions. This phenomenon has been observed on many stall controlled rotors; the difference between highest and lowest stall levels (above rated wind speed) has been observed to be of 25 to 35 %. The aim of the Joule Project 'Multiple Stall Levels' is to determine the causes of this behavior and subsequently suggest design and/or operational rules to control such comportment. The project has started with the analysis of existing observations, available to the partners in the project and a screening of the literature on the subject. One interesting item that has been identified is the relatively small change in measured flap loads compared to the changes in measured power.

A Nordex N30/300 turbine with Aerpac APX 29 blades is being instrumented for a dedicated measurement program that will start shortly. The observation of stall patterns on the blades, with the aid of the 'Stall Flag* technique forms an essential part of this measurement program. In preparation, a list of hypotheses concerning multiple stall has been established, accompanied by a list of observables that need to be recorded for the rejection or the substantiation of the different hypotheses. The paper describes the progress made in the project in some detail.

Keywords

-